A Data Science Approach to Automated Raman Spectra Analysis

Dominic Conricode

# Scope

- Define an algorithmic approach for fitting Raman spectra with a series of peaks

  - Extract parameters that correlate to graphite microstructural properties

- Use this technique to characterize graphite grades over a range of temperature values

# Background and Motivation

- Graphite is used in MSRs, FHRs, and HTGRs for various purposes (and non-nuclear applications)

  - Important to understand characteristics of different graphite grades to choose the most beneficial one for a given application

- Semi-automatic fitting does exist, but it requires a human to oversee and manually adjust when it gets “stuck”

- A fully automated approach would allow for analysis at a larger scale

# Approach

## Pre-Processing

- Input: CSV file(s) from Raman spectroscopy with “x” and “y” values

Raw Input Data

- Smooth data with Savitsky-Golay filter

  - Parameters for smoothing can be passed in by user

- Normalize spectrum (easier to make normalized initial guesses)

- Restrict “x” values to between 500 and 3000

  - The peaks we are interested in are known in the literature to not be outside this range

Savitsky-Golay filter applied to raw data with a window-length of 27 and a 2nd degree polynomial fitting

- Remove background noise by fitting baseline (done automatically)

- Iterative baseline algorithm:

  - Fit a polynomial to the entire spectrum

  - Remove data points above the fit line

  - Re-fit to new spectrum with less data points

  - Repeat until the error between fit and data is within threshold or less than a certain number of points is getting removed between iterations

Fitting baseline with a 2nd degree polynomial

## Fitting

- Uses scipy.optimize.minimize package to minimize residuals between reconstructed spectrum and real spectrum

- Initial Guesses: (Gaussian/Lorentzian peaks can be defined in terms of peak center, amplitude, and area) (User can select which peak type to use)

  - Center: peaks known to be in given range from literature, choose middle value of this range

  - Amplitude: 1 (subject to change?)

  - Area: 1 (subject to change?)

- Loss function (error):

  - Sum the curves at each “x” value

  - Distance between reconstructed curve and real curve

- Constraints

  - Area of G’_3Da / G’_3Db approximately 0.5

    - Known from literature

  - Center of G’_3Db  > G’_2D (these peaks’ center range overlaps, other peaks don’t have this issue)

MAPE 0.0088

# Results (In Progress)

- Run this program on many graphite spectra spanning a wide range of grades and temperatures

# Analysis (In Progress)

- What can the extracted parameters tell us about the graphite

# Summary of Research

- Fit spectra, extract parameters, characterize graphite

# Summary of Research Experience

- Learned optimization techniques

- …